[              D A N C A D  3 D   V E R S I O N   2 . 6              ]

SUPPLEMENTAL UPDATE INFORMATION FOR

D A N C A D 3 D . C O M  v 2 . 6  (tm)

Information for registered v2.5H users about changes in v2.6x.

COPYRIGHT (c) 1994 by DANIEL H. HUDGINS, ALL RIGHTS RESERVED

DANCAD3D, 466 DIAMOND STREET, SAN FRANCISCO, CA  94114, U.S.A.

This documentation file may not be edited, altered, changed, or

translated without express written permission from its author.

Read the disclaimer in the documentation files DANCAD3D.DOC

and CAMPLOT.DOC before using the programs.

Use the program PRINTDOC.EXE on the DANCAD3D  shareware  disks  to

print  out  this  file.  This  update information is included only for

users upgrading to v2.6 who have registered and purchased the DANCAD3D

user outfit.  The information given will  only  be  understandable  to

users  who  have  the  full  outfit.  If  you are a new shareware user

please read the files DANCAD3D.DOC and CAMPLOT.DOC that  are  included

on the shareware disks.

[UPDATE26.DOC-1]

[              D A N C A D  3 D   V E R S I O N   2 . 6              ]

Thank you for purchasing the v2.6 update disks,  or for purchasing

the revised v2.5 outfit with the v2.6 program disks.  The  information

below  briefly describes the changes made to the programs.  In general

carefully reading the menus and prompts in the programs will be of the

most help in using the new features.

As much a possible I have kept the menus in v2.6 the same as  v2.5

to  minimize  the  need  to re-learn commands.  The programs have been

changed considerably behind the menus,  and tens of thousands of lines

of code altered or completely rewritten.

DANCAD3D  v2.6A  is  the  successor  to  DANCAD3D v2.5H.  DANCAD3D

version 2.6 is the first to allow working on drawings  of  "unlimited"

size,  so  long  as  you have free space (up to 32MB) on your harddisk

DANCAD3D will expand the size of the workspace to allow drawings  with

hundreds  of  thousands  of  line segments.  The amount of free RAM no

longer limits the number of  line  segments  available  (although  the

program may work a little faster if you have more that 512KB RAM free,

depending on the type of drawing you are working on.)

DANCAD3D v2.6A adds these features:

* Virtual memory, make drawings up to 32MB with only 512KB RAM.

* Up to more than 750000 line segments available for drawing.

* Up to 999 separate elements in the workspace at one time.

* Digitizer tablet support for absolute distance drawing & tracing.

* Jockey menu of commands added to drawing editor.

* String variables added to macro language.

* Preserve workspace option on quit to DOS, fast way to go out to DOS.

* Much of the underlying program code has been modified.

* Files load and save Industry menus added.

* Directly reads HP-GL type plot files from other CAD programs.

* Directly writes HP-GL, DXF, PostScript (tm), and G & M code files.

DANCAM v2.60 and DANPLOT v2.60 add these features:

* Joy-Stick support added to jog and teach menus.

* Teach mode to make ASCII tool path files, with curve fitting.

* Large digital position read out (DRO) in jog and teach modes.

* Scratch pad, relative, and absolute position read out on DRO.

* Angle and total distance measurements in jog and teach modes.

* Software digital noise filter on home switch inputs.

* Ramping supported in home up to home switches command.

* Improved timing of feed rate control.

* Improved motor testing utilities.

* DANPLOT v2.6 will directly read HP-GL type files.

* Rotation of C axis in DANPLOT v2.6 can be reversed.

* Printed circuit pattern macro for Bi-Polar stepper motor driver.

Below  I will take up each of the listed improvements and add some

additional information where needed.

[UPDATE26.DOC-2]

[              D A N C A D  3 D   V E R S I O N   2 . 6              ]

CHANGES MADE TO DANCAD3D

In order to make DANCAD3D more useful I have rewritten much of the

program to use virtual memory,  thereby,  for the most part,  removing

the  problem  of  the  "out of lines" error (so long as you have space

free on your harddisk.) Digitizer tablet  support  makes  the  program

useful  for tasks that are not possible to do without a tablet,  since

the tablet allows accurate tracing and measurements.  The  Files  Save

and Load Industry sub-menus make importing and exporting files easier.

VIRTUAL MEMORY

Because  so much of DANCAD3D's code has been changed some commands

may work a little slower or faster.  In many cases I  had  to  include

more  than  one  method  of  operation of the commands to optimize the

execution speed depending of the amount of free  RAM  memory  and  the

location of the drawing elements within DANCAD3D's workspace.

In  previous versions of DANCAD3D the workspace was limited by the

amount of RAM installed in your computer.  DANCAD3D  v2.6  allows  you

make  drawings are larger than your DOS RAM because DANCAD3D v2.6 will

automatically swap data in and out of your system RAM to and from your

harddisk such that up to 32MB of data can be held in the workspace.

Each line segment in DANCAD3D v2.6 takes 40  bytes  so  32MB  will

allow  drawings  with up to about 750000 line segments.  DANCAD87 v2.6

takes 52 bytes for each line segment,  and so will allow about  600000

line segments.  Since most drawings have less than 50000 line segments

you now have more than enough capacity.

Because  the  workspace is larger than the amount of memory in the

DOS system (640KB) DANCAD3D v2.6 needs to make temporary files on your

harddisk to hold parts of the drawing.  DANCAD3D  v2.6  requires  that

you  have  free  space on your harddisk at all times while DANCAD3D is

being used.  A disk full error will probably result  in  the  loss  of

some  drawing  data  since  that  command  will  not  be able to write

temporary files.  For most users,  if you have 2MB or 3MB free on your

harddisk  when  you  enter  DANCAD3D  v2.6  the  program  will operate

satisfactorily.  If you  know  that  you  are  going  to  work  on  an

especially  complex  drawing  you  can  make  sure  that you have more

harddisk space free.

Always quit DANCAD3D back to the DOS prompt before you  turn  your

computer off,  otherwise the workspace file will not be closed and you

will have "lost clusters" on  your  harddisk.  If  you  have  a  power

failure  while running DANCAD3D you will need to run the DOS CHKDSK /F

command  to  recover  the  lost  clusters  (be  sure  to  delete   the

FILE0???.CHK  file(s) created by CHKDSK in order to free the recovered

disk space.)

The number of elements available in DANCAD3D's  workspace  at  one

[UPDATE26.DOC-3]

[              D A N C A D  3 D   V E R S I O N   2 . 6              ]

time  has  been  increased from 256 in v2.5 to 999 in v2.6.  The value

was kept to 999 since a higher value would have mandated changing  the

file  naming  convention for the Elements file type.  If you find that

you need to add additional elements beyond 999 you can  use  the  Join

command to consolidate some elements already in the workspace.

The  Block  text  command can now be used with any font to do page

layout since you will not run out of lines (so long as you  have  free

space  on  the  harddisk.)  This is important if you want to make tool

path files for DANPLOT that have a lot of text, i.e.  engraved plaques

and such.

In  the small menu that comes up over DANCAD3D's opening screen an

option has been added to allow you to change the  path  that  will  be

used  to open and save the workspace file.  You can have more than one

workspace preserved if you change the workspace path  before  entering

DANCAD3D's  main  menu.  If  you  attempt to run more than one copy of

DANCAD3D on your system (under a multitasking environment)  each  copy

of  DANCAD3D  should  be  in  a  different sub-directory to avoid file

conflicts in the many temporary files created.

When you go from DANCAD3D's main menu to the Write  command  (text

editor)  the  program  no longer asks you if you want to save or abort

the elements in the workspace,  since the workspace is on the harddisk

all the time you are using the program.

If  you  have several megabytes of RAM memory in your computer and

you can create a RAM disk, the workspace file can be re-located to the

RAM disk to speed up some operations when drawings are large enough to

occupy more than one "chunk" in  the  workspace.  Re-locating  all  of

DANCAD3D  to  a  RAM  disk will result in more speed improvement,  but

would require a RAM disk of about 4MB or more.  Remember that the  RAM

disk  vanishes  when you turn your computer off,  and so any files you

wish to retain need to be saved to your harddisk before you turn  your

system off.

When  you  quit DANCAD3D to DOS you now have three options,  Save,

Preserve, and Abort.  Save and Abort work as in v2.5.  Preserve allows

you to quit DANCAD3D without erasing the workspace file,  you can then

quickly reenter DANCAD3D to continue working on the drawing.  Preserve

works  just fine so long as you do not decrease the amount of free DOS

RAM on your system between the time you  quit  DANCAD3D  and  run  the

program  again.  If  you  are going to decrease the amount of free DOS

RAM use the Save option to preserve  the  workspace  (save  is  slower

since each element will be in a separate file, which can be up to 1000

files if you have 999 elements in the workspace.)

DIGITIZER TABLET

Digitizer  tablet  support  has  been  added for absolute distance

drawing and tracing.  I chose the add support for the CalComp  Drawing

Board  II  series  of  tablets.  The  drawing  editor  cursor  can  be

[UPDATE26.DOC-4]

[              D A N C A D  3 D   V E R S I O N   2 . 6              ]

controlled for drawing to absolute distances,  and the buttons on  the

tablet  "cursor"  or  pen  can be used to activate some commands.  The

tablet works well on my system,  but since this is a new feature I  do

not know if there are any compatibly problems with other computers, if

you  are going to buy a tablet for use with DANCAD3D try to try it out

first with DANCAD3D to see if it works on your computer,  be  sure  to

follow the set-up instructions carefully.

A "fee hand" drawing mode has been added for tablet users to trace

and draw curves while one of the "cursor" buttons is held down.

With  DANCAD3D  v2.6 you can use a CalComp "Drawing Board II" (tm)

type digitizing tablet to make drawings or trace drawings.  The tablet

measures distances in an absolute way,  and so can be used for  making

measurements of angles and distances,  on drawings or photographs,  by

using DANCAD3D's measure command (in the drawing editor.)

The tablet must be turned on and set up before you  run  DANCAD3D.

To  activate the tablet control of the drawing cursor press [F2] while

in the drawing editor,  or select [T]ablet from the  drawing  editor's

[S]et-up menu.  Do not try to turn the tablet use on if the tablet and

serial  port  have not been configured properly,  since activating the

tablet without the tablet being ready will make the program lock up.

The tablet supersedes the mouse & cursor keys in  the  control  of

the  drawing  cursor.  To  return control of the drawing cursor to the

mouse or cursor keys press [F2] again.  Use the  [F2]  key  to  change

between the tablet and the mouse & cursor keys while drawing.

Read  the help screen in the [F2] menu for a listing of the tablet

"cursor" button  functions.  "Cursors"  with  up  to  16  buttons  are

supported.  The  one  or  two  button  pens,  as  well as the 4 button

cursors can also be used.

To set up your tablet for use do these things:

1.  Connect the tablet to the serial port and then turn tablet on.

2.  Run external DOS command C:\DOS\MODE COM1:9600,N,8,1,P

3.  Run CalComp driver: C:\CALDB\CCMOUSE.COM

4.  Run CalComp utility: C:\CALDB\RMDRV.EXE

5.  Click the tablet cursor on the "Config/Exit" mark on  the  top  of

the tablet,  the  tablet  should  beep  once  and  the  power  and

configuration lights will go from dark to lighting up.

6.  Put the tablet cursor on the "Bank/B"  mark  on  the  top  of  the

tablet,  the  power  light  should  be out,  then click the cursor

button, the tablet will beep and the power light will come on.

7.  Click the tablet cursor on the "Configuration/Macros-#18" mark  on

[UPDATE26.DOC-5]

[              D A N C A D  3 D   V E R S I O N   2 . 6              ]

the top of the tablet,  the tablet will beep and the  power  light

will  go  out.  If  the light comes on,  click again,  because the

whole point of fiddling with the tablet configuration is  to  turn

"B-18" off.

8.  Click  the  tablet cursor on the "Save-#1-Default" mark on the top

of the tablet, the tablet will beep twice and the power light will

flash.

9.  Click  the  tablet  cursor on the "Config/Exit" mark on the top of

the tablet, the tablet should beep and the power and configuration

lights will become dark.

10. Run   DANCAD3D v2.6,  select [D]raw,  select [F]ront view,  select

[D]raw,  press  [F2],  select  the  use of the tablet.  The tablet

cursor  should  then  move  the  drawing  cursor  around,  if  not

something  is  wrong.  If  the  tablet  "cursor"  goes outside the

tablet active area the tablet power light will blink, and DANCAD3D

will "go dead" until you put  the  tablet  "cursor"  back  in  the

tablet active area.  Depending on how you have the zoom set in the

drawing  editor,  the  drawing  cursor  can go off the side of the

screen when you move the tablet "cursor" around (if you cannot see

the drawing cursor on the screen zoom out with the [-] & [*]  keys

until you see what area the tablet covers in your drawing.)

Steps  5 through 9 only need to be done once since the tablet will

remember the setting of its configuration "buttons",  i.e.  #B18 being

set  off.  Steps  2  through 4 can be put in your AUTOEXEC.BAT file so

they will automatically be done when  your  computer  is  powered  on.

Step  1 will not be a problem,  remembering to turn on the tablet,  if

you have your computer plugged into a power strip with  a  switch,  so

that  the  power supply for the tablet will be turned on when you turn

the power strip for your computer on (leave the power  switch  on  the

tablet on all the time.)

So all you will have to do normally is press [F2] while drawing in

DANCAD3D, all the other steps are done automatically by the tablet and

your computer after you set them up once.

The  "button" Bank B:  #18 is used to set the "CTS Line Enable" to

the state 0/OFF.  You will probably also want to set "button" Bank  B:

#10  to  state  0/OFF  to  turn off the tablet pen click off,  because

DANCAD3D v2.6 makes a click AFTER the button press is detected,  where

as the tablet clicks before DANCAD3D v2.6 has read the tablet, and may

cause  you to let the button up on the tablet "cursor" before DANCAD3D

v2.6 knows the button was pressed.  The click DANCAD3D v2.6 makes when

the tablet "cursor" button is pressed can be selected  from  the  menu

that comes up when you press [F2] in the drawing editor.

Recommended settings for the tablet configuration would be:

[UPDATE26.DOC-6]

[              D A N C A D  3 D   V E R S I O N   2 . 6              ]

CONFIG # 1  2  3  4  5  6  7  8  9 10 11 12 13 14 15 16 17 18

BANK A   1  1  0  1  0  1  0  0  1  1  0  0  0  1  0  0  0  1

BANK B   0  0  1  1  1  1  0  0  0  0  0  0  0  0  0  0  1  0

MODE  can  be set to COM2 if COM1 is being used for your mouse and

you have the tablet connected to the COM2  connector  on  your  serial

port  board.  The  executable  files CCMOUSE.COM & RMDRV.EXE come on a

floppy disk with your tablet.  See Appendix B in the  CalComp  Drawing

Board II User's Guide,  Configurations,  for the meaning of the tablet

settings.

DANCAD3D v2.6 uses data format "CalComp 9100 Format #1",  which is

an  ASCII  data  format  with  a resolution of 0.001 inch.  The actual

accuracy of the tablet is about 0.01 inch, so although the tablets can

give smaller values in other data formats the  extra  resolution  only

makes  it  harder  to  hold  the  tablet  cursor  still  enough to get

particular position values.  In the tablet menu,  you get by  pressing

[F2]  in  DANCAD3D's  drawing  editor,  you  can  change  the  minimum

increment that the drawing cursor will move as  you  move  the  tablet

"cursor".

DANCAD3D  v2.6  should  automatically  adjust  to the size of your

tablet (tablets come in different sizes from 12" by 12" to  table  top

size  for  large  drawings.)  You  can  change the center point of the

tablet relative to DANCAD3D's workspace center to enable you  to  work

on  drawings  larger  than  your  tablet  (see  the tablet menu in the

drawing editor.)

If you are using a tablet with DANCAD3D please  write  to  me  and

tell  me.  Knowing  how  many  users  are  using a tablet will help me

decide how much need there  is  to  expand  the  tablet  interface  to

include additional features.

For  general  drawing using a track-ball or mouse will probably be

more useful since DANCAD3D has special processing of the  mouse  input

to  make  drawing  straight lines easier.  Processing the tablet input

would defeat the purpose of using an absolute position  input  device.

If  you  can afford both a mouse/track-ball and tablet they complement

each other because the mouse/track-ball is good for  general  drawing,

and the tablet is necessary for tracing accurately.

JOCKEY MENU

A  new  menu  has been added to DANCAD3D's drawing editor to allow

some new drawing editing commands that users have asked for.

The [J]ockey element menu was added to DANCAD3D in version 2.6A to

add some additional manipulations of elements.  The  [M]edian  command

lets  you  center  an  element on any axis or set of axis,  and on the

element center or the workspace center.  The [F]it-size  command  lets

you have an element fit a "rectangular" volume, e.g.  if the volume is

not the same proportions as the element, the element will be distorted

[UPDATE26.DOC-7]

[              D A N C A D  3 D   V E R S I O N   2 . 6              ]

to fit.  [R]everse reverses the order  of  the  line  segments  in  an

element,  and  so  does not change the appearance of the element,  but

does let you have an element plot in  reverse  and  so  is  useful  in

preparing CNC/CAM tool paths.  The [L]evel command lets you "level" an

element.  The  [L]evel command is useful to "level" an element created

by using a digitizer tablet to trace a drawing or  photograph  (it  is

nearly  impossible  physically  to tape a photo level to 0.001" on the

tablet,  so use [L]evel later and you do not need  to  fuss.)  [L]evel

works  by having you pick two points that line up on the horizontal or

vertical, misalignment up to 44.9 degrees is allowed. [S]plit lets you

"break" an element into two parts  by  grouping  the  lines  into  two

elements,  the appearance of the element does not change.  The [F]ind-

points command in the [L]ines sub-menu can be useful  in  finding  the

line segment to [S]plit the element at.

New  macro  commands have been added to correspond to the commands

added to the Jockey menu:

# number MEDIAN center mode

WHERE: number = Element number to act on.

center = C or Z

mode   = X, Y, Z, F, S, T, or A

# number FIT_SIZE mode x1 y1 z1 x2 y2 z2

WHERE: number = Element number to act on.

mode   = I or A

x1y1z1 = First corner point of volume to fit.

x2y2z2 = Other corner point of volume to fit.

# number REVERSE

WHERE: number = Element number to act on.

LEVEL currently codes as a macro ROTATE command.

# number SPLIT line

WHERE: number = Element number to act on.

line   = Workspace line number to be last line of first part.

The jockey commands should be used as the last step if you act  on

more  than  one  element.  You may need to reset the element center(s)

after using MEDIAN or FIT_SIZE if you are  going  to  edit  individual

elements some more.

[UPDATE26.DOC-8]

[              D A N C A D  3 D   V E R S I O N   2 . 6              ]

MACRO COMMANDS FOR NEW FILE TYPES

Some  new macro commands have been added for the Industry standard

file formats, to load an HP-GL file:

LOAD HPGL filename.ext mode scale

WHERE: mode   = 0 to 3, 0=HPGL center, 1=centered, 2=0+90

scale  = 1 for inch, or any value to change size.

To save a HPGL file to disk the special driver  HPGL.STF  is  used

with  the PLOT command.  When the PLOT command uses a *.STF driver the

installed port or filename is replaced with the current value  of  the

string  variable  with the name FILENAME.USE,  so the name of the file

you want to create needs to be assigned to FILENAME.USE before you use

the PLOT command with the *.STF driver(s).

LET$ FILENAME.USE = "YOURFILE.HPG"

# 0  PLOT 0 0 0 -1.0E+18 240 1 0 0 DRIVERS\HPGL.STF

The drivers POSTSCRIPT.STF and DXF.STF work the same way.

LET$ FILENAME.USE = "YOURFILE.PS"

# 0  PLOT 0 0 0 -1.0E+18 240 1 0 0 DRIVERS\ADOBE_PS.STF

LET$ FILENAME.USE = "YOURFILE.DXF"

# 0  PLOT 0 0 0 -1.0E+18 240 1 0 0 DRIVERS\DXF.STF

Two other DXF save commands  are  available,  one  for  saving  2D

projections,  and  one for saving 3D vectors.  The DXF2D file uses the

same parameters as the DANCAD3D 2D-REAL file type.

# 0  SAVE DXF2D 0 0 0 -1.0E+18 240 1 0 0 YR2DFILE.DXF

# 0  SAVE DXF3D YR3DFILE.DXF

To save G&M code files use the Files Save Industry G-code  Install

menu to configure, then the macro command:

# 0  SAVE GCODE YOURFILE.G

The new macro commands will automatically be coded into your macro

file when you use the macro automatic Output feature.

STRING VARIABLES

DANCAD3D  v2.6  adds  support  for  string  variables to the macro

language (v2.0 through v2.5H have supported numeric variables.) String

variables are used to substitute  for  the  parameters  to  the  macro

commands  that  are  not  numbers.  The  primary  use  of  the  string

variables would be to allow you to input filenames, and pass lettering

[UPDATE26.DOC-9]

[              D A N C A D  3 D   V E R S I O N   2 . 6              ]

to the LETTERING command.

New macro commands to work with strings  are:  LET$,  INPUT$,  and

OUTPUT$.  LET$  creates  and  initializes  a  string  variable  so the

variable can used.  INPUT$ lets the  user  enter  a  string  from  the

keyboard.  OUTPUT$  displays  the contents of a string variable on the

video screen (OUTPUT$ can be  used  as  a  replacement  for  the  ECHO

command.)

One  subtlety  of using the string variables needs extra emphasis.

When you enter just the variable name (without a preceding $) you  are

passing  the  variable  name to the command literally,  you would only

pass the literal variable name  to  the  few  commands  that  use  the

variable  name  to  act  on  the  variable.  If  the  variable name is

preceded by a $ symbol then the contents of the variable are passed to

the macro command.  It is possible to have a string  variable  contain

the  name  of  another  string  variable and so there may be instances

where either form would be permissible, and so the program cannot tell

if you want the name of the variable or the contents of  the  variable

without you explicitly telling the program which one you mean by using

the $ symbol.

String variables are saved as individual disk files,  the variable

name can be a full filename including a path.  String variables can be

saved to a RAM disk to  speed  up  access,  e.g.  E:\VARNAME.1.  Since

string  variables  are  filenames  be  careful  that you do not use an

important filename for a string variable name.  You can have  as  many

string variables as your disks will hold.

String constants are put between two quotation marks, e.g. "SOME".

It  is  possible  to  combine string variables and string constants by

using the square brackets,  e.g.  [ "A B" + $SV1 + $SV2 + " C D.  " ].

If you do not want a string and one is called for by the macro command

you  can  pass  a  null string with "".  If a string constant needs to

contain a quotation mark you can follow  the  quotation  mark  in  the

string  with  a percent symbol and the macro processor will ignore the

quotation mark in the middle of the string,  e.g.  ""%WOW!"%"  assigns

"WOW!" to the string, where as "WOW!" would be WOW!.

EXAMPLE: LET$ STRVAR.123 = "This is "%special"% case."

In  order  to  bring  up  a  file  directory  a pre-defined string

"variable" name DIR is in the macro environment.  By using $DIR  "*.*"

you can get a file directory to come up and when you select a filename

the  selected  filename will be returned like the contents of a string

variable.  The string ("*.*") following the $DIR  is  a  default  file

mask used to select the filenames displayed in the directory.

EXAMPLE: LET$ varname = $DIR "*.*"

The  LET$ command must be used before a string variable is used to

initialize the variable.

[UPDATE26.DOC-10]

[              D A N C A D  3 D   V E R S I O N   2 . 6              ]

LET$ varname = varvalue

LET$ varname = $DIR "mask"

LET$ varname = $DIR $varname

The varname is a filename.  The  varvalue  can  be  either:  a  string

constant  ("S"),  the  contents of a string variable ($VARNAME),  or a

string statement between square brackets ([ "This " + "that." ]).  The

mask, used with the $DIR feature,  is any file mask like one used with

the DOS DIR command ("*.*",  "*.MAC",  "FONTS\*.FON", "?????SET.???").

When the string variable contains the name of a file or a  file  mask,

the  varname  can  be  used  as the default mask with the $DIR feature

(remember to initialize the variable that holds the default path).

The INPUT$ command lets the user enter strings while  a  macro  is

running.  To  make menus you would use the LOCATE,  ECHO,  and OUTPUT$

commands along with the INPUT$ command.

INPUT$ varname default

The varname is  a  filename.  The  second  parameter,  default,  is  a

default  string  you  want  displayed.  The  default  can  be a string

constant ("SOMETHING"),  or a variable,  including the variable  being

assigned.  The  statement  INPUT$ VARNAME.4 $VARNAME.4 is valid if the

variable has been previously assigned, and therefore is already on the

harddisk.  The user can edit the string line of  text  displayed  with

the usual cursor keys,  back-space,  [Ins],  and [Del].  The string is

passed and the macro proceeds when the [Return] key is pressed.

EXAMPLE: TEXT

LOCATE 1 2

ECHO Enter string: %%

INPUT$ VARNAME.4 ""

The OUTPUT$ command is normally used to display the contents of  a

string variable on the video screen.

OUTPUT$ $varname

The varname is a filename that was used as a string variable.

EXAMPLE: LOCATE 1 3

LET$ VARNAME.4 = [ "The string is: " + $VARNAME.4 ]

OUTPUT$ $VARNAME.4

If  you  want to have a string variable converted into a lettering

element in the drawing workspace (to be included as part of a drawing)

you can use the VAR$ operator as the  text  string  parameter  to  the

macro  LETTERING  command.  The VAR$ operator is described in the v2.5

CAD manual in Appendix B as part of the LETTERING command description.

To use VAR$,  you replace the text constant (on a macro code text line

by  itself)  with  VAR$,  and  place  the name of the string variable,

preceded by a dollar sign, i.e.  $ (ASCII code number 36) on the macro

[UPDATE26.DOC-11]

[              D A N C A D  3 D   V E R S I O N   2 . 6              ]

code  text  line  immediately  below,  the  other  parameters  to  the

lettering command are placed on the macro code text  line,  or  lines,

below  the name of the string variable.  Putting the $ symbol in front

of the string variable name tells the macro processor  that  you  want

the contents of the variable used for the text.

; BEGIN EXAMPLE MACRO CODE USING VAR$

INIT

LET$ FONTNAME = "FONTS\*.FON"

; You can skip the initialization of the

; variables after the macro is run once.

LET$ LINE1 = "R.A.R. 2479"

LET$ LINE2 = "EI150,1-27-94"

LET$ LINE3 = "TWIN PEAKS"

TEXT

LOCATE 1 2

OUTPUT$ "Enter text line #1: "

INPUT$ LINE1 $LINE1

LOCATE 1 3

OUTPUT$ "Enter text line #2: "

INPUT$ LINE2 $LINE2

LOCATE 1 4

OUTPUT$ "Enter text line #3: "

INPUT$ LINE3 $LINE3

LOCATE 1 6

OUTPUT$ "Enter font name...: "

INPUT$ FONTNAME $DIR $FONTNAME

LOCATE 1 8

OUTPUT$ "<Working>"

LETTERING

VAR$

$LINE1

0 -1.70 0  0 0 0  0.9 0.8

1 25 16 0    0 1 0.333333333 7

C J $FONTNAME

LETTERING

VAR$

$LINE2

0 0 0   0 0 0    0.9 0.8

1 25 16 0    0 1 0.333333333 7

[UPDATE26.DOC-12]

[              D A N C A D  3 D   V E R S I O N   2 . 6              ]

C J $FONTNAME

LETTERING

VAR$

$LINE3

0 1.70 0   0 0 0  0.9 0.8

1 25 16 0    0 1 0.333333333 7

C J $FONTNAME

; END EXAMPLE MACRO CODE USING VAR$

The IF THEN GOTO command has been extended to include strings.  To

compare  strings use the IF$ THEN GOTO command.  Operators = < > <= >=

and <> are allowed.

EXAMPLE: IF$ $STRVAR1 >= $STRVAR2 THEN GOTO :LABEL123

As with all  other  aspects  of  the  macro  language  key  words,

operators, and parameters need to be separated by a space character.

INDUSTRY FILE MENUS

Two  new  menus  were added to DANCAD3D from DANCAD3D's main menu.

The menus are Files Load Industry, and Files Save Industry.  DANPLOT's

main menu option #2 was also expanded to support HPGL files.

HPGL FILES

DANCAD3D v2.6 can read and write HPGL files,  see the  Files  Load

and Save Industry sub-menus.  You can use other CAD type programs that

make HPGL files that contain only HPGL commands PA,  PR,  PU,  PD, and

SP,  to make drawings that DANCAD3D can load.  Use the other program's

plotter  driver  for  HP74XX or HP75XX type HP plotters,  and have the

data go to  a  disk  file,  rather  than  the  plotter  port.  Scanned

drawings can be converted to HPGL data by using a third-party "Raster-

to-Vector" program.  Four, loading the HPGL drawing, options are given

for  the  centering  of the loaded data.  To alter the window clipping

limits,  DANCAD3D uses when HPGL  files  are  saved,  edit  DANCAD3D's

HPGL.STF driver, with the Hardcopy Plotter Install command.

DXF FILES

DANCAD3D v2.6 can write line segments to a DXF type data file, see

the  Files  Save Industry sub-menu.  Options for saving the data in 2D

perspective or 3D data format are given.

POSTSCRIPT (tm) FILES

[UPDATE26.DOC-13]

[              D A N C A D  3 D   V E R S I O N   2 . 6              ]

DANCAD3D v2.6 can write line segments to a PostScript (tm) drawing

file,  see the Files Save Industry sub-menu.  The Laser command in the

Hardcopy sub-menu is set up to output PostScript (tm) (using the *.LZR

drivers)  or  HPGL/2  (using  the  LJIII???.PLT  drivers)  to  a laser

printer.

G & M CODE FILES

DANCAD3D v2.6 can write line segments to a G code file for  making

CNC  tool  path files,  see the Files Save Industry sub-menu.  M codes

can also be installed to be included in the file made.  The feed  rate

setting can be configured to correspond to different line colors found

in  the  drawing element being saved as G codes.  If you would like to

see DANCAD3D also read G code files then please send me some example G

code files,  of the type you want to see read,  on 1.2MB floppy  disk,

and I will look at the files you send,  to see how hard it would be to

include reading G code files.

CHANGES MADE TO DANCAM AND DANPLOT

DANCAM and DANPLOT can now create  tool  path  files  as  well  as

execute tool path files.  The jog menu has been altered to support the

joy  stick,  and  display  position  in large numbers.  The home up to

switches has been improved to support ramping and have a digital noise

filter to correct for noise in the switch contacts.  The configuration

menus have been altered and improved.  DANPLOT will now directly  read

HPGL files, or convert HPGL files into ASCII files.

JOY STICK SUPPORT

DANCAM and DANPLOT can now use the joy stick to move one,  two, or

three  motors  at  once.   The  joy  stick  is  installed   with   the

configuration  menu  option #1,  (board type.) If you have a joy stick

that has a throttle control the throttle can operate the  z  axis.  If

your  joy  stick  does  not  have a throttle control you can connect a

second joy stick whose "Y" axis will control the z axis.

The joy stick port pin out is given  in  the  current  CAMPLOT.DOC

file  on  the  DANCAM/DANPLOT shareware disks.  You can wire up a high

quality joy stick using the pin out information if you want  something

better  than  the available game type PC joy sticks.  If you use a joy

stick extension cable be sure that the cable has all the pins wired or

else the z axis may not work.

In the teach mode the buttons on the joy stick work like the [Ins]

and [Del] keys in DANCAD3D's drawing editor.  When teaching in DANPLOT

the tool should be down while marking lines,  since DANPLOT tool  path

files only contain the line segments for the tool down condition.

Be  sure that the joy stick centering controls are at their center

[UPDATE26.DOC-14]

[              D A N C A D  3 D   V E R S I O N   2 . 6              ]

point when configuring the joy stick so that the joy stick  will  have

its  proper  full  range  when  used.  You may need to fiddle with the

centering controls, and redo the configuration,  to get the full range

from  +100  to -100.  You should use a good quality joy stick to avoid

problems with the stick flopping off center.

Reversal of the joy stick direction orientation  is  available  in

the  configuration  menu  (to make up for the motor direction.) If you

have servo motors on your machine, an option to have the motor ramping

off in the jog and teach commands is available,  since only  with  the

ramping  off can the motors reach top speed while jogging with the joy

stick (because the program does not know when you will let up  on  the

joy  stick.) When jogging with servo motors and the ramping off in the

jog and teach modes you should "let up" on the  joy  stick  slowly  to

avoid having the motors overshoot.

TEACH MODE

You can now jog the tool (or work-piece) around to mark  out  line

segments  for  an  ASCII  tool  path file.  The tool path created with

DANCAM or DANPLOT can be edited with DANCAD3D.

You can exit the teach mode and reenter the teach  mode  later  to

append  additional  line segments.  The motor power must be on and the

tool at the "home" reference point when you enter the  teach  mode  in

order for the motion to be repeated accurately later.

The  way  to  make  a  tool  path  file  is to jog the tool to the

starting point of a line segment (tool motion) and click the joy stick

button #1 then move the tool to the other end of the line segment  and

click  the  joy stick button #2.  You can also use the [Ins] and [Del]

keys (and the cursor keys) on  the  key  board  to  save  lines.  This

"drawing"  of  the  lines  works just like drawing lines in DANCAD3D's

drawing editor.  The orientation of the y axis (or the others) may  be

reversed  from  DANCAD3D  depending  on  how you installed the scaling

factors in DANCAM or DANPLOT.

You can make curves by activating the curve fitting option of  the

teach  mode.  The curve fitting option lets you mark a few points on a

curve,  then the curve fitting  option  smoothes  out  the  curve  and

appends  the  smooth  curve to the open tool path file you are saving.

The maximum cord length on the curve made is user selectable.

You  can  change the "line color" sent to the ASCII file while you

are in the teach mode, see the menu of commands.

LARGE DIGITAL READ OUT

In the jog and teach modes you can have  the  measurement  of  the

position  displayed  with  large numbers to make the display easier to

read from across the room.  The read out can measure absolute from the

[UPDATE26.DOC-15]

[              D A N C A D  3 D   V E R S I O N   2 . 6              ]

zero point (like the center of DANCAD3D's workspace.),  from the  home

position  (which  is  set  up  in  DANCAM's or DANPLOT's configuration

menu),  and from the "scratch pad" reference point.  The  scratch  pad

reference  point  can  be  reset  while  in  the  jog  command to make

movements relative to the tools  current  position  when  the  scratch

value  is  made  zero.  In  the  teach  command  the from "mark" point

reading gives you the distance from the marked point or last point  of

a  line segment saved.  The "jog" measurement is the movement relative

to the position the tool was in when you entered the jog  screen,  you

can  exit  and re-enter the jog command and the position read out will

be correct as long as the motors and electronics have not changed.

When the,  jog or teach,  small size numbers  display  is  on  the

screen  you can now read the angles of,  and the total distance along,

the line being measured (or "drawn.")

In the jog and teach commands you can now enter  the  absolute  or

relative  coordinate  values  ([G]oto)  for a desired position and the

motors will have the tool travel to that point.

To have the readings displayed be accurate you need  to  have  the

tool  at  home,  and the motors on,  before you enter the jog or teach

commands.  When the backlash compensation is used,  for it to  operate

properly you need to keep in mind what direction the first move is in,

just  like  starting  the plotting of a file (so that the "slack" does

not get taken up the wrong way.)

HOME SWITCHES

Some improvements to the home switch operation have been  made.  A

digital  noise  filter  has  been  added to compensate for intermitent

false signals that can arise from micro switches.  The  digital  noise

filter  can  also  help  if what triggers the switch has a tendency to

"sag" back so that the switch changes state without the motor making a

step (which might happen from vibration or motor  overshoot.)  If  the

motor speed ramping is active the motors will now "ramp up" until they

reach  full  speed  or a home switch toggles.  Having the motors "ramp

down" is not possible since the  location  of  the  home  switches  is

"unknown," which is why the home switches are used,  i.e.  to find the

home position.  Since the travel to home does not ramp down,  if servo

motors are used, the home switches should have enough "over travel" to

compensate  for the overshoot (when the final move toward home is made

the speed is reduced to minimize the affect  of  overshoot.)  A  small

display  has  been added during the "move home to switches" to display

the status of the home switch inputs on the parallel port,  as an  aid

in debugging your hook up.

FEED RATES

The  timing procedure has been improved to give more accurate feed

rate for any value of the delay entered.

[UPDATE26.DOC-16]

[              D A N C A D  3 D   V E R S I O N   2 . 6              ]

MOTOR TESTING

The motor setting commands have been improved.  The ramping is now

turned off while in  motor  test  options  #1,  #2,  &  #3,  to  avoid

erroneous results.  The backlash compensation is also defeated so that

the  motor  will  stop  at the starting "position" on tests #2 and #3.

Motor test #2 has been expanded to help catch motor resonance problems

that might allow the motor to pass the test with too small of a p.w.f.

value.  When you use motor test #2 is is  generally  a  good  idea  to

increase  the (minimum) value of the p.w.f.  found by about 20 percent

to insure a margin of safety.  In general when using servo motors  the

p.w.f.  would  be  set to 0 and the feed rate controlled with the line

color to feed rate delay configuration.  A menu option has been  added

to  the  motor testing menu to allow the p.w.f.  to be changed without

doing the tests.

HPGL FILE INPUT

DANPLOT can now read  a  sub-set  of  HPGL  commands  directly  to

control the motors.  The commands supported are PA,  PR,  PU,  PD, and

SP.  The SP ("select pen") command is converted to  the  current  line

color,   and   therefore  will  control  the  relays  and  feed  rate.

Conversion of SP to line color is equivalent,  e.g.  pen  #2  is  line

color  #2,  pen  #16  is  line color 16,  and so on.  DANPLOT can also

convert HPGL files into ASCII files, which would improve throughput if

a tool path was to be executed repeatedly under some conditions.  HPGL

support is useful  to  read  plot  files  made  by  "raster-to-vector"

conversion programs, and drawing files made in CAD programs other than

DANCAD3D.

PRINTED CIRCUIT PATTERN FOR BIPOLAR TRANSLATOR BOARD

Users  have  asked  that I provide a printed circuit pattern for a

inexpensive yet powerful stepper motor translator  driver  board  that

they  could  build.  So  on  the  DANCAM v2.6 & DANPLOT v2.6 shareware

disks I have included a DANCAD3D v2.6 macro file  that  will  generate

the drawing files for the printed circuit board traces,  the component

placement drawing,  and the hole location  drawings.  This  "BIPOLAR2"

board  is  a design improvement of the "ARRANGEMENT2" drawing shown in

the DANCAD3D v2.5H CAM manual.

The parts list and brief assembly instructions are  given  in  the

file  CAMPLOT.DOC  file  on the DANCAM & DANPLOT shareware disks.  You

can transfer the pattern to a blank circuit board with the  "iron  on"

dry  copier  (laser printer) toner transfer sheets now available.  You

can print out the patterns using DANCAD3D  on  any  of  the  supported

printers  or  plotters.  I  will  probably  have  photo-copies  of the

patterns and perhaps some additional information  available  to  users

who write (for a nominal fee.)

[UPDATE26.DOC-17]

[              D A N C A D  3 D   V E R S I O N   2 . 6              ]

COMPATIBLY PROBLEMS

Changes to way some of the commands in DANCAD3D v2.6  operate  may

cause  macros (particularly automatic output macros) made with v2.5 to

not produce the same result.  The LATHE,  THICKEN,  WINDOW,  and PURGE

commands  in  v2.6  may  make elements with a different number of line

segments.  The problems arise if the DELETE,  or ALTER  line  commands

are  used  in  the  same  macro,  since  DELETE,  or ALTER make direct

reference to the line number in the workspace array,  which  would  be

different when the macro is run under v2.6.  Macros created under v2.6

should  run  on v2.6 without problems.  In general this is not a major

problem for most users,  but you may have  to  edit  the  line  number

references  in  some  macros  created on v2.5 if you want to run those

macros on v2.6.  Compatibility can be achieved  if  you  run  the  old

macro under v2.5H then save v2.5H's workspace as set of Elements, then

load the elements into v2.6 with the automatic output macro turned on,

then continue editing the drawing in v2.6 (this fix works because v2.6

is loading whole elements rather than making line references), the new

v2.6  output  macro  would  be  used  in  place of the v2.5H macro for

appending additional macro commands from v2.6.

You should set  the  file  buffers  in  your  CONFIG.SYS  file  to

BUFFERS=80,  when  you  use  v2.6,  to  speed up access to the overlay

files.

SUGGESTIONS

Please  also  read  the  files DANCAD3D.DOC and CAMPLOT.DOC on the

v2.6 disks since those documentation  files  are  revised  to  include

information  about the programs in their current implementation.  Feel

free to write to me about improvements you think would  be  useful  to

you in your use of the programs.

[UPDATE26.DOC-18]