DCMotors Program Documentation:

Program requirements:

DCMotors requires an IBM PCAT (a 80286 with 80287) or better with a VGA

graphics display card (the graphics mode used is 640 by 480 by 16 colors)

and a DOS 3.1 or later operating system.  The user can make hard copies of

the motor performance graph using the P command (see below) with an Epson

compatible printer.

Operating Instructions:

To run DCMotors type DCMOTORS <Return>.  To run DCMotors with an input

data file type DCMotors FileName <Return> (where FileName is the name of

the ASCII text data file).

When running the DCMotors without an input file the user will be prompted

for the following motor parameters:

Vs : power supply voltage in Volts   (not optional, must be entered)

Ke : voltage constant in Volts/KRPM  (optional, may not be entered)

Kt : torque constant in In*Oz/Amp    (optional, may not be entered)

Fs : static drag friction in In*Oz   (optional, may not be entered)

Fd : damping friction in In*Oz/KRPM  (optional, may not be entered)

Rc : total path resistance in Ohms   (optional, may not be entered)

If any of the above parameters are not known enter a null line by pushing

<Return>.  If any null lines are entered DCMotors will automatically prompt

the user for additional information later on in an attempt to determine the

missing information by solving a system of simultaneous equations using one

or more motor power supply Voltages and corresponding motor RPMs.  The user

will be prompted for Kt only if a null line is entered for Ke (since Ke and

Kt are numerically related only one of them needs to be entered).  DCMotors

can handle from one to two unknows from this list: Ke (and Kt), Fs, Fd, Rc.

Due to the nonlinear nature of the performance equations it is not possible

to solve for more than two unknowns.  Some restrictions are placed on some

of the known motor parameters when solving for unknown parameters (the user

is informed about the nature of problem if no solution is possible with the

user supplied parameters).  All motor parameters must be positive in value

and only Fs and Fd are allowed to be zero (the user will be automatically

prompted for new information if these conditions are not observed).

After the motor performance graph is displayed the user can either make a

hard copy by pushing "P" (Print graphics on a printer), write all the motor

parameters into an ASCII text data file for later use by pushing "S" (Save

motor data into a text file) or exit DCMotors by pushing any other key.

Performance Graphs:

The four sides of the graphics display have different scales indicating

the motor parameters being plotted.  Each parameter scale factor is color

coded to the corresponding parameter curve.  The locations and colors are:

Location    Color    Parameter

Bottom      White    Current (in Amps)

Top         Cyan     Torque (in In*Oz)

Left        Green    Velocity (in RPM)

Right       Yellow   Power (in Horse Power)

Right       Red      Efficiency (in percent)

The Green curve displays motor velocity vs current (use the bottom scale

for Amps and the left scale for RPM).  The Green curve is actually composed

of three straight lines; one line segment goes from zero supply current and

maximum RPM (operating the motor as a tachometer) to maximum supply current

and zero RPM (motor stalled) while the two other line segments indicate the

no-load supply current and the corresponding RPM.

The Cyan curve displays motor torque vs velocity (use the top scale for

In*Oz and the left scale for RPM).  The Cyan curve is actually composed of

two straight lines; one line segment goes from the unloaded RPM to zero RPM

(motor stalled) while the other line segment indicates the stalled torque.

Although motor torque is proportional to supply current both are plotted in

different colors and with different scale factors for convenience.  The two

scale factors are individually selected to be "round" which means that the

two curves are generally not coincident.

The Red curve displays motor efficiency vs current (use the bottom scale

for Amps and the right scale for %).  The Red curve is actually composed of

three different line segments; one curved segment is the actual efficiency

while the two straight line segments indicate the peak efficiency and the

corresponding current drain.

The Yellow curve displays motor power vs current (use the bottom scale

for Amps and the right scale for HorsePower).  The Yellow curve is actually

composed of three different line segments; one curved segment is the actual

power while the two straight line segments indicate the peak power and the

corresponding current drain.

Error Messages:

Enter a positive number.

A non-numeric value has been entered.

Numeric value must be above zero.

A negative value has been entered.

Same Voltage as motor data point [N].

A non-unique power supply voltage has been entered.

Same RPM as motor data point [N].

A non-unique motor RPM has been entered.

Motor Voltage and RPM must be above zero.

A negative value has been entered for Voltage or RPM.

Invalid solution value for [param : value].

Poorly chosen input data has resulted in an badly conditioned solution

matrix.  No valid solution is possible for the given parameter value.

Try rerunning DCMotors and entering slightly different input values.

Unable to solve for [param1] and [param2] when [param3] is zero.

No valid solution is possible for parameter1 and parameter2 with the

given parameter3 input value.  Try rerunning DCMotors and entering a

different input value for parameter3.  If parameter3 is "denominator"

no valid solution is possible for the given parameter input values.

Try rerunning DCMotors and entering slightly different input values.

Too many unknowns for a unique solution.

An attempt was made to solve for more than two unknowns.  DCMotors can

only handle up to two unknowns.

Motor stalled: static drag friction too high.

Fs (static drag friction) is too high for the given power supply voltage

for the motor to start rotating.  Try rerunning DCMotors with a reduced

value for Fs or an increased value for Vs or Kt.

Unable to find motor data file [FileName].

The specified ASCII input data file wasn't found.  Try rerunning DCMotors

with different input file name or a path name to an existing input file.

Unable to create motor data file [FileName].

Insufficient space on the disk to create an ASCII output data file.  Try

rerunning DCMotors in a different partition or remove some files from the

existing partition.

Micheal D. Cranford